| Components: - Three (3) × 2 m beam struts - One (1) × 1 m beam strut - Two (2) × end units with attached shackles and hooks - Two (2) × lifting slings – 6m long - Two (2) × crane forks - One (1) × torque wrench - One (1) × 30mm socket - One (1) × breaker bar - One (1) × 30mm spanner/podger - Twenty-Four (24) × nuts/bolts & washers |  |
| --- | --- |

| Step 1 – Remove Components from Stillage - Remove the stillage locking bars - Using a crane or telehandler, remove the crane forks from the stillage Step 2 – Lay Out Beam Struts - Lay beam struts end-to-end in correct configuration - use flats on each flange to prevent rolling |  |
| --- | --- |

| Step 3 – Bolt Beam Connections - Ensure all flange faces are clean and free from debris - Bolt all beam connections using supplied hardware Bolting Requirements: - Four (4) bolts per connection - Tighten to 150 Nm using supplied torque wrench - ❌ DO NOT OVER-TIGHTEN THE BOLTS |  |
| --- | --- |

| Step 4 – Attach Lifting Slings - Attach one lifting sling through each large top shackle - Ensure all shackle pins are securely tightened Step 5 – Connect to Crane Hook - Attach slings directly to the crane lifting hook - ❌ Do not use additional slings or chains |  |
| --- | --- |

| Step 6 – Attach Crane Forks - Hoist beam clear of the ground - Attach swivel hooks directly to the sprung arm on top of crane forks - Control lines may be fitted to assist lift control Step 7 – Positioning Into Panel Pack - ⚠️ CRITICAL SAFETY STEP - Ensure load is always balanced - Fully insert crane forks - Use supplied ladders to assist fork insertion - ❌ Do not climb onto delivery vehicles |  |
| --- | --- |

| Disassembly and Storage - Carefully disassemble the Pack Boy™ - Return all components to the storage stillage (see image in step 1 for storage) - Secure components for carrier collection Fastener Removal: - ❌ Do not use the torque wrench - Use supplied breaker bar only | Conditions of Hire and Use ⚠️ MANDATORY CONDITION - Any wilful or negligent damage, or loss of components, will result in charges for repair or replacement |
| --- | --- |